# Road Recorder
Laptop Command Guide

Windows PowerShell ↔ Raspberry Pi 4 • USB storage • Video review • Quality tuning

This guide is tailored to the current Road Recorder setup. Commands run on the laptop are marked WINDOWS POWERSHELL; commands run after connecting to the Pi are marked RASPBERRY PI SSH.

# 1. Your current setup

Raspberry Pi address last confirmed: 10.182.60.135 (may change when the hotspot/router reconnects).

Pi username: omgawale

Project on USB: /mnt/road-recorder-usb/road-recorder/Road-Recorder

USB recordings: /mnt/road-recorder-usb/recordings

USB metadata: /mnt/road-recorder-usb/metadata

USB logs: /mnt/road-recorder-usb/logs

Camera: IMX219; current configured capture is 1280×720 at 30 FPS, H.264, 5,000,000 bit/s (5 Mbps).

Important: use the current Pi IP address if it changes. Do not format disks or delete recordings while following this guide. The separate 1 TB external HDD should remain untouched.

# 2. Connect from the Windows laptop

## A. Open PowerShell

On Windows, open Start, type PowerShell, and launch Windows PowerShell or Terminal. Make sure the laptop and Pi are on the same Wi-Fi/hotspot.

## B. Connect by SSH

ssh omgawale@10.182.60.135

Type yes if SSH asks to trust the Pi host key for the first time, then enter the Pi account password when prompted. Password characters may not appear while typing.

## C. Check the Pi and USB

Run these commands after SSH connects:

hostname
whoami
ip -br address
findmnt /mnt/road-recorder-usb
df -h /mnt/road-recorder-usb

Exit the SSH session when finished:

exit

# 3. Start and control Road Recorder

Run these commands inside the SSH session, not in the Windows PowerShell prompt.

cd /mnt/road-recorder-usb/road-recorder/Road-Recorder
python3 main.py

Use the application menu: 1 = Start Recording, 2 = Stop Recording, 3 = refresh status, 4 = list recordings, 5 = USB diagnostics, 7 = exit. Stop cleanly from the menu before disconnecting the USB.

## Check whether processes are running

pgrep -af 'main.py|rpicam-vid'

## Check the latest logs

ls -lt /mnt/road-recorder-usb/logs/ | head -10
tail -50 /mnt/road-recorder-usb/logs/recording.log

## List today's recordings

ls -lh /mnt/road-recorder-usb/recordings/$(date +%F)/

## Check metadata index

cat /mnt/road-recorder-usb/metadata/recording_index.jsonl

# 4. Copy a video from the Pi to the laptop

Keep a separate Windows PowerShell window open (not the SSH window). Create a folder on the laptop:

New-Item -ItemType Directory -Force "$HOME\RoadRecorderVideos"

Copy MP4 files to the laptop. Use the actual date folder shown on the Pi; the example below uses 2026-10-13. Replace that date when copying newer recordings.

scp "omgawale@10.182.60.135:/mnt/road-recorder-usb/recordings/2026-10-13/*.mp4" "$HOME\RoadRecorderVideos\"

scp "omgawale@10.182.60.135:/mnt/road-recorder-usb/recordings/2026-10-13/*.mp4" "$HOME\RoadRecorderVideos\"

Then open File Explorer and go to the RoadRecorderVideos folder in your Windows user folder.

## Copy one specific video

scp "omgawale@10.182.60.135:/mnt/road-recorder-usb/recordings/2026-10-13/RR_20261013_233254_001.mp4" "$HOME\RoadRecorderVideos\"

Use the real filename shown by ls; filenames and dates will change for future recordings.

# 5. View video and inspect quality

## On the laptop

Open the copied MP4 in VLC or the Windows Movies & TV/Media Player app. Copying the MP4 to the laptop is usually easier than trying to play a large video directly over SSH.

## On the Pi: inspect duration, resolution, codec, and size

ffprobe -v error -show_entries format=duration,size:stream=codec_name,width,height,r_frame_rate,bit_rate -of default=noprint_wrappers=1 "/mnt/road-recorder-usb/recordings/2026-10-13/REPLACE_WITH_FILENAME.mp4"

## Check for decoding errors

ffmpeg -v error -i "/mnt/road-recorder-usb/recordings/2026-10-13/REPLACE_WITH_FILENAME.mp4" -f null -

If FFmpeg returns to the prompt without printing errors, that is a good sign. These commands read the video; they do not modify it.

# 6. Reduce future video size while keeping visual quality

Current setting is 5 Mbps at 1280×720 and 30 FPS. Start with a modest change to 4 Mbps; it may reduce file size by roughly 20% for similar footage, but the exact result depends on road motion, foliage, lighting, and encoder behavior. Test a short sample and compare it on the laptop before committing.

First inspect the current config:

grep -n -A 12 -B 3 "camera:" /mnt/road-recorder-usb/road-recorder/Road-Recorder/config.yaml

Make a backup of the configuration before editing:

cd /mnt/road-recorder-usb/road-recorder/Road-Recorder
cp config.yaml config.yaml.backup-before-bitrate-test
grep -n "bitrate" config.yaml

Edit config.yaml with nano:

nano config.yaml

Find the camera bitrate line, currently expected to be bitrate: 5000000, and change it to bitrate: 4000000. Keep resolution 1280×720, FPS 30, and codec h264 unchanged. Save in nano with Ctrl+O, Enter, then exit with Ctrl+X.

Restart Road Recorder for the setting to take effect, then record a 1–2 minute sample and compare the image on the laptop. Do not expect old videos to shrink; this setting affects new recordings only.

## Restore the original config if needed

cd /mnt/road-recorder-usb/road-recorder/Road-Recorder
cp config.yaml.backup-before-bitrate-test config.yaml

Do not use FFmpeg to overwrite the original MP4. If you later want to compress old videos, write to a new filename and compare quality first.

# 7. Useful USB and camera checks

## Confirm USB mount and free space

findmnt /mnt/road-recorder-usb
df -h /mnt/road-recorder-usb
lsblk -o NAME,SIZE,FSTYPE,LABEL,MOUNTPOINTS

Expected: the recording target is mounted at /mnt/road-recorder-usb from the SanDisk USB partition. Do not run mkfs, fdisk write commands, or any disk-formatting command.

## Check camera detection

rpicam-hello --list-cameras

If that command is unavailable or behaves differently on your OS, use the project diagnostics and inspect the latest logs instead.

# 8. Optional: download logs to the laptop

scp "omgawale@10.182.60.135:/mnt/road-recorder-usb/logs/recording.log" "$HOME\RoadRecorderVideos\"

You can then open recording.log in Notepad or VS Code on the laptop.

# 9. Troubleshooting

SSH cannot connect: Check that laptop and Pi are on the same network, then verify the Pi IP on its screen or from a known working Pi session with: hostname -I

Permission or path error while copying: Check the exact filename and date folder on the Pi with: ls -lh /mnt/road-recorder-usb/recordings/$(date +%F)/

No MP4 appears: Stop recording cleanly from the menu, then inspect the latest log. Do not unplug the USB while recording.

USB is not mounted: Do not start recording and do not create files in the mount path as a workaround. Run findmnt and the application's USB diagnostics first.

Bitrate change seems to have no effect: Confirm the app loads this config.yaml, restart the application after editing, and inspect a new file with ffprobe. Do not judge using a recording created before the change.

# 10. Quick reference

Laptop PowerShell: ssh omgawale@10.182.60.135
Pi: cd /mnt/road-recorder-usb/road-recorder/Road-Recorder && python3 main.py
Pi: ls -lh /mnt/road-recorder-usb/recordings/$(date +%F)/
Pi: tail -50 /mnt/road-recorder-usb/logs/recording.log
Laptop PowerShell: scp "omgawale@10.182.60.135:/mnt/road-recorder-usb/recordings/2026-10-13/*.mp4" "$HOME\RoadRecorderVideos\"

Safety reminder: stop recording cleanly before disconnecting USB power or storage. This guide does not require formatting any drive or touching the 1 TB external HDD.